Purnima ma’am notes  computer network

**📘 CHAPTER 5 – MULTIPLE ACCESS PROTOCOL AND NETWORKS

(Detailed Notes – BCA Sem 3 | Networking)**

1. Introduction to Multiple Access

In computer networks, a communication channel is often shared by multiple users. When many devices try to transmit data over the same channel, collisions and interference may occur.
To avoid this, specific rules or protocols are used. These are called Multiple Access Protocols.

Why Multiple Access is needed?

- Ensures fair access to the medium.

- Reduces collisions and retransmissions.

- Helps in efficient usage of channel bandwidth.

- Coordinates transmission among multiple stations.

Multiple Access techniques are essential in LANs, wireless networks, Ethernet, and cable communication systems.

2. Types of Multiple Access Protocols

Multiple access protocols are mainly classified into:

A. Random Access Protocols

Stations transmit randomly without reservation.

Examples:

- ALOHA

- Slotted ALOHA

- Carrier Sense Multiple Access (CSMA)

- CSMA/CD (Collision Detection)

B. Controlled Access Protocols

Access is regulated by a central controller.

Examples:

- Polling

- Token passing

C. Channelization Protocols

Channel is divided among users.

Examples:

- FDMA

- TDMA

- CDMA

→ For your syllabus, the main focus is CSMA/CD and Ethernet.

3. CSMA/CD (Carrier Sense Multiple Access with Collision Detection)

(Most important topic)

CSMA/CD is a network protocol used in Ethernet LANs to control access to the shared channel.

Meaning of CSMA/CD

- CS = Carrier Sense:
Each device listens to the channel before sending data. Transmission starts only if channel is free.

- MA = Multiple Access:
Many devices share the same communication medium.

- CD = Collision Detection:
If two devices transmit simultaneously, collision occurs. Devices detect collision and stop transmission.

3.1 Steps of CSMA/CD Protocol

- Carrier Sensing:

  - Station checks whether the medium is idle.

  - If busy, it waits until it becomes free.

- Transmission:

  - When the medium is free, the station sends data.

- Collision Detection:

  - While sending, the station keeps listening to detect collision.

  - If a collision occurs (signal energy becomes abnormal), the station immediately stops.

- Jam Signal:

  - To inform all other stations, a short “jam signal” is broadcasted.

- Backoff Algorithm (Binary Exponential Backoff):

  - After collision, stations wait for a random time.

  - Waiting time increases exponentially after each collision.

- Retransmission:

  - After the delay, station tries again.

3.2 Features of CSMA/CD

- Used in wired Ethernet (half-duplex mode).

- Not used in full-duplex switched Ethernet.

- Works best for short distances and low traffic.

- Prevents continuous collisions.

3.3 Limitations

- Cannot be used in wireless networks (Wi-Fi uses CSMA/CA instead).

- Performance decreases with heavy traffic.

- Used only in older Ethernet setups.

4. Ethernet LANs (IEEE 802.3)

Ethernet is the most widely used LAN technology. It uses CSMA/CD in its traditional form.

4.1 Features of Ethernet

- Follows IEEE 802.3 standard.

- Uses Star topology (modern) or Bus topology (old).

- Data rate ranges:

  - 10 Mbps, 100 Mbps, 1 Gbps, 10 Gbps, 40 Gbps, 100 Gbps

- Supports unicast, multicast, broadcast.

4.2 Ethernet Frame Format

An Ethernet frame contains:

- Preamble (7 bytes) – synchronization

- SFD – Start Frame Delimiter (1 byte)

- Destination MAC Address (6 bytes)

- Source MAC Address (6 bytes)

- Length/Type (2 bytes)

- Payload/Data (46–1500 bytes)

- CRC (4 bytes) – error detection

4.3 Advantages of Ethernet

- Simple and cheap

- Highly reliable

- Easy to install and expand

- Supports multiple speeds

- Interoperable with many devices

5. Connecting LANs and Backbone Networks

To build bigger networks, multiple LAN segments must be connected. Various networking devices are used for this.

5.1 Repeaters

- Operate at OSI Layer 1 (Physical Layer)

- Regenerate and amplify signals

- Extend the physical distance of LAN

- No filtering of traffic

- Forward everything (even errors)

Limitation: Cannot reduce congestion; creates a single collision domain.

5.2 Hubs

- Multi-port repeater

- Operate at Layer 1

- Broadcasts input signal to all ports

- All devices share a single collision domain

- Increases collisions

Not used in modern networks.

5.3 Bridges

- Operate at Layer 2 (Data Link Layer)

- Understand MAC addresses

- Forward frames to correct segment based on MAC address

- Use learning table for intelligent forwarding

- Reduce traffic and collisions

Advantages

- Increase network performance

- Divide LAN into segments

- Filter unwanted traffic

5.4 Switches

A switch is an advanced multi-port bridge.

Features

- Operates at Layer 2

- Each port is a separate collision domain

- Supports full-duplex

- Uses MAC address table

- No collisions in modern Ethernet

Advantages

- Very fast

- Secure

- Efficient

- Supports VLANs

Switches are the backbone of modern Ethernet LANs.

5.5 Routers

Routers operate at the Network Layer (Layer 3).

Functions

- Forward packets based on IP addresses

- Connect different networks

- Build & maintain routing table

- Support routing algorithms (RIP, OSPF, BGP)

Uses

- Connecting LAN to WAN

- Internet connectivity

- Packet filtering

- Traffic management

5.6 Gateways

Gateways are the most intelligent network devices.

Features

- Operate from Layer 4 to Layer 7 (Application Layer)

- Connect completely different types of networks

- Perform protocol conversion

Examples

- VoIP Gateway

- Email Gateway

- Cloud Gateway

6. CSMA/CD vs Switches (Important Concept)

| Feature | CSMA/CD | Switch |
| --- | --- | --- |
| Mode | Half duplex | Full duplex |
| Collision | Yes | No |
| Efficiency | Low | High |
| Medium | Shared | Dedicated |
| Use now | Rare | Very common |

7. Important Definitions (Short Notes)

1. Collision Domain

A network area where frame collisions can occur.
Hubs & repeaters share a single collision domain; switches break collision domains.

2. Broadcast Domain

A network area where broadcast messages reach.
Routers break broadcast domains.

3. Backbone Network

A high-speed central network connecting many LANs, switches, and routers.

Here are well-organized, clean, and exam-ready notes of Unit 6 based entirely on the files you uploaded (Module 6 – Networking).
I have rewritten everything in simplified language, added headings, bullet points, examples, and removed repetition so you can study easily.

📘 UNIT 6 – Network Layer Functions & Protocols (Detailed Notes)

1. ROUTING

❖ What is Routing?

Routing is the process of selecting the best path for data to travel from a source to a destination across networks.

- Performed by Routers (Layer 3 device).

- Works at:

  - OSI Model → Network Layer

  - TCP/IP Model → Internet Layer

- Router forwards packets using:

  - Packet header information

  - Forwarding table (Routing Table)

  - Routing Algorithms + Metrics

2. ROUTING METRICS

Routing metrics are factors used by routing algorithms to choose the best path.

Common Metrics

| Metric | Meaning |
| --- | --- |
| Hop Count | Number of routers a packet passes. Fewer hops = better. |
| Delay | Time taken for packet processing + transmission. Lower delay = better. |
| Bandwidth | Capacity of the link (bps). Higher bandwidth = better. |
| Load | How busy a router or link is. Lower load = better. |
| Reliability | Stability of the link (failure rate, error rate). Higher reliability = better. |

3. TYPES OF ROUTING

A. Static Routing

- Routes are manually added by the network administrator.

- No automatic updating.

Advantages

- No CPU overhead.

- No bandwidth usage.

- More secure.

Disadvantages

- Not suitable for large networks.

- Manual configuration is time consuming.

- Requires expert knowledge.

B. Default Routing

- Router sends all packets to one default gateway, regardless of network.

- Useful when:

  - There is only one exit point.

  - A small network uses a single upstream router.

C. Dynamic Routing

- Routes are automatically updated based on network changes.

- Uses routing protocols like RIP, OSPF.

Features

- Routers update each other about topology changes.

- Automatically finds new paths.

Advantages

- Easy to configure.

- Automatically adapts to failures.

Disadvantages

- Uses more bandwidth & CPU.

- Less secure.

4. CLASSIFICATION OF ROUTING ALGORITHMS

1. Adaptive Routing (Dynamic)

Routing decisions change according to topology and traffic.

Types:

- Isolated

  - Router uses only its own information.

  - Example: Hot Potato Routing.

- Centralized

  - One central device has full network info.

  - Example: Link-State Algorithm.

- Distributed

  - Routers share info with neighbors.

  - Example: Distance Vector Routing.

2. Non-Adaptive (Static)

Routes are fixed and do not change.

Types:

- Flooding

  - Packet is sent on all outgoing links except the incoming one.

  - Causes duplicates but guarantees delivery.

- Random Walk

  - Packet is forwarded to a randomly chosen neighbor.

  - Highly robust.

3. Hybrid Routing

Combination of both adaptive + non-adaptive.

Types:

- Link-State Routing

- Distance Vector Routing

5. ROUTING PROTOCOLS

1. RIP (Routing Information Protocol)

- Uses hop count to choose shortest path.

- For small networks.

2. IGRP (Interior Gateway Routing Protocol)

- Developed by Cisco.

- Supports up to 100 hops.

- Uses multiple metrics:

  - Load, reliability, bandwidth.

3. EGP (Exterior Gateway Protocol)

- Used to share routing info between different autonomous systems.

4. EIGRP (Enhanced IGRP)

- Cisco proprietary.

- Faster, more reliable than RIP and IGRP.

- Classless routing.

5. OSPF (Open Shortest Path First)

- Link-state protocol.

- Uses Dijkstra's SPF algorithm.

- Good for large networks.

6. BGP (Border Gateway Protocol)

- Used on the Internet.

- Manages routing between autonomous systems.

- Uses AS numbers (ASN).

6. DIFFERENCE: Routing vs Flooding

| Routing | Flooding |
| --- | --- |
| Needs routing table | No routing table needed |
| May give shortest path | Always gives shortest path |
| Less reliable | More reliable |
| Less traffic | High traffic |
| No duplicate packets | Duplicate packets occur |

7. NETWORK PROTOCOLS IN NETWORK LAYER

A. FUNCTIONS OF THE NETWORK LAYER

- Logical Addressing (IP Address)

- Host-to-Host Delivery

- Fragmentation – Breaking large packets into smaller ones.

- Congestion Control – Avoiding packet overload.

- Routing & Forwarding

B. IMPORTANT NETWORK LAYER PROTOCOLS

1. IP (Internet Protocol)

- Connectionless.

- No delivery guarantee.

- Two versions:

  - IPv4 (32-bit, dotted decimal)

  - IPv6 (128-bit, hexadecimal)

2. ARP (Address Resolution Protocol)

- Converts IP address → MAC address.

How ARP Works

- Host broadcasts ARP request with target IP.

- Device with that IP sends back its MAC address.

- MAC address stored in ARP cache.

Types:

- Static

- Dynamic

3. RARP (Reverse ARP)

- Converts MAC address → IP address.

- Used by diskless systems to know their IP.

4. ICMP (Internet Control Message Protocol)

- Used for error reporting and diagnostics.

- Example: Ping uses ICMP Echo Request & Reply.

Messages:

- Error messages (destination unreachable, time exceeded)

- Query messages (echo request/reply)

5. IGMP (Internet Group Management Protocol)

- Used for multicasting.

- Helps hosts join or leave multicast groups.

8. INTERNET PROTOCOLS (Overview)

Common Internet Protocols:

- TCP/IP

- SMTP

- PPP

- FTP

- SFTP

- HTTP

- HTTPS

- TELNET

- POP3

- IPv4, IPv6

- ICMP

- UDP

- IMAP

- SSH

9. IP ADDRESSING

- A unique address assigned to each device.

- Used to identify computers on a network.

- Two types: IPv4, IPv6