# Unit 5

CSMA/CD Protocol

1. Full Form

CSMA/CD → Carrier Sense Multiple Access with Collision Detection

Used mainly in wired Ethernet (IEEE 802.3).

2. What is CSMA/CD?

CSMA/CD is a network access method that allows many devices to share the same communication channel (like a common bus cable).
Before sending data, a device first checks if the channel is free.
If two devices accidentally send data at the same time → collision happens → both stop and retransmit after a random time.

3. Why is CSMA/CD used?

- To avoid collisions on a shared medium.

- To detect collisions if they occur.

- To ensure efficient channel utilization.

4. Working Principle

CSMA/CD works using three steps:

(A) Carrier Sense

Computer listens to the channel.

- If channel is idle, it can transmit.

- If channel is busy, it waits.

(B) Multiple Access

Many devices can access the same medium.

(C) Collision Detection

While sending data, the device also monitors the medium.
If it detects interference (collision), it stops sending immediately.

5. CSMA/CD Steps (Very Important for Exams)

- Sense the channel (check if free).

- If free → start transmitting.

- While transmitting → listen for collision.

- If collision detected →

  - Stop transmission immediately.

  - Send a JAM signal (to inform others).

- Wait for a random backoff time (Binary Exponential Backoff).

- Try sending again.

6. Collision Handling

JAM Signal

- A special signal broadcast to alert all devices that a collision has happened.

- Makes all devices stop sending.

Binary Exponential Backoff

- After collision, device waits a random time before retransmitting.

- Random wait time doubles after every collision.

Example:
1st collision → wait between 0–1 slot
2nd collision → wait 0–3 slots
3rd collision → wait 0–7 slots

7. Where is CSMA/CD Used?

- Used in Wired Ethernet (10Base5, 10Base2, 10Base-T).

- Works only in half-duplex networks.

- Not used in modern full-duplex switches.

8. Why CSMA/CD is NOT used now?

Because modern Ethernet uses switches and full-duplex, where collisions cannot occur.

9. Advantages

✔ Simple and easy to implement
✔ Reduces chances of collision
✔ Efficient for light traffic
✔ Works well in small networks

10. Disadvantages

✘ Cannot prevent collisions completely
✘ Performance decreases when traffic is high
✘ Not suitable for wireless networks (hidden node problem)
✘ Not used in full-duplex networks

11. CSMA/CD vs CSMA/CA (short difference)

| CSMA/CD | CSMA/CA |
| --- | --- |
| Used in wired Ethernet | Used in Wi-Fi (wireless) |
| Detects collision | Avoids collision |
| Works in half-duplex | Works in full-duplex wireless |
| Not used anymore | Still used in WLAN |

Ethernet LANs

1. What is Ethernet?

Ethernet is the most widely used LAN (Local Area Network) technology.
It defines how devices communicate over a wired network.

It follows the standards of IEEE 802.3.

2. Key Features

- Wired LAN technology

- Uses CSMA/CD (in old shared Ethernet)

- Very high speed (10 Mbps → 100 Mbps → 1 Gbps → 10 Gbps → 100 Gbps)

- Uses MAC addresses for communication

- Supports star, bus, and extended star topology

- Reliable and easy to maintain

3. Ethernet LAN Components

(a) Network Interface Card (NIC)

Allows a device to connect to Ethernet.

(b) Switch / Hub

- Hub: older, shared medium → collisions occur

- Switch: modern device, collision-free, full-duplex

(c) Ethernet Cables

- Coaxial cable (old)

- Twisted pair (UTP/STP)

- Fiber optic (for high-speed links)

(d) Ethernet Frames

Basic unit of communication.

4. Ethernet Frame Format (Important for exams)

| Preamble | SFD | Destination MAC | Source MAC | Type/Length | Data | CRC |

Explanation:

- Preamble: 7 bytes to sync receiver

- SFD (Start of Frame Delimiter): 1 byte

- Destination MAC: 6 bytes

- Source MAC: 6 bytes

- Type/Length: 2 bytes

- Data: 46–1500 bytes

- CRC: 4 bytes for error checking

5. Types of Ethernet LANs (Based on Speed)

1) Standard Ethernet

- 10 Mbps

- IEEE 802.3

- Cable: Coaxial, UTP

2) Fast Ethernet

- 100 Mbps

- IEEE 802.3u

- Cable: UTP (Cat 5)

- Topology: Star

3) Gigabit Ethernet

- 1000 Mbps (1 Gbps)

- IEEE 802.3z/802.3ab

- Cable: Fiber or UTP (Cat 5e, Cat 6)

4) 10 Gigabit Ethernet

- 10 Gbps

- IEEE 802.3ae

- Cable: Fiber & UTP (high category)

5) 40/100 Gigabit Ethernet

- Very high-speed backbone networks

- Used in data centers

6. Ethernet Topologies

1) Bus Topology (Old)

- All devices connected to one cable

- Used in 10Base2, 10Base5

- Collisions happen → uses CSMA/CD

2) Star Topology (Current)

- Devices connected via switch

- No collisions

- High performance

3) Extended Star

- Many switches connected together

7. Ethernet Addressing

MAC Address

- Unique 48-bit address

- Written in hex (e.g., 00:A2:4B:11:FF:C9)

- Used for identifying devices in LAN

8. Ethernet Transmission Modes

- Half-duplex: devices take turns; collisions possible.

- Full-duplex: both send & receive simultaneously; no collisions.

Modern Ethernet uses full-duplex.

9. CSMA/CD in Ethernet

- Used only in old shared hubs

- Not used with switches (collision-free)

Steps:

- Sense channel

- Transmit

- Detect collision

- Backoff

- Retransmit

10. Types of Ethernet (Based on Media)

Copper-based Ethernet

- UTP (Cat 5, Cat 6, Cat 7)

- Cheaper, easy to install

Fiber-optic Ethernet

- Very high speed

- Long-distance communication

- Immune to noise

11. Advantages of Ethernet

✔ Simple and inexpensive
✔ High-speed connectivity
✔ Easy expansion
✔ Reliable and scalable
✔ Fewer errors
✔ Supports full-duplex communication

12. Disadvantages

✘ Limited to 100 meters (UTP cables)
✘ Wired setup → less mobility
✘ Older versions suffered collisions
✘ Network performance drops with old hubs

13. Applications

- Colleges, offices

- Home networks

- Data centers

- Industrial networks

- Server rooms

14. Short Notes Summary (Very Useful for Exams)

Ethernet

A family of LAN technologies defined by IEEE 802.3 offering high-speed wired communication using MAC addressing and Ethernet frames.

Fast Ethernet

100 Mbps Ethernet using star topology and UTP cables.

Gigabit Ethernet

1 Gbps speed Ethernet used in modern networks and switches.

Ethernet Frame

Structured packet containing source MAC, destination MAC, data, and CRC.

1. Connecting LANs

Sometimes a single LAN is not enough for an organization (college, office, multi-floor building).
So, multiple LANs are connected for:

- Greater coverage

- More users

- Better performance

- Centralized network management

Connecting LANs is done using networking devices like:
✔ Repeaters
✔ Hubs
✔ Switches
✔ Bridges
✔ Routers

2. Backbone Networks

A Backbone Network is the central, high-speed network that connects several smaller LANs together.

Think of it as the main highway and the LANs as small roads connecting to it.

Features of Backbone Network

- Very high data rate (1 Gbps, 10 Gbps, 40 Gbps)

- Connects multiple LANs on different floors or buildings

- Used in universities, corporate offices, and data centers

- Reduces congestion in local networks

Types of Backbone Networks

1) Distributed Backbone

Each floor has a hub/switch connected to the main cable.

Floor Switch 1 ----|

Floor Switch 2 ----|---- Main Backbone Cable

Floor Switch 3 ----|

2) Collapsed Backbone (Most Common)

A central switch/router connects all LANs.

LAN 1 ----\

LAN 2 ----- Central Backbone Switch

LAN 3 ----/

3) Parallel Backbone

More than one backbone connection for reliability.

3. Repeater (Important Small Question)

Definition

A Repeater is a device used to regenerate and amplify weak signals in a network so they can travel longer distances.

Why Repeaters are Needed?

Over long distances, signals become weak or distorted due to noise and attenuation.
A repeater cleans the signal and then forwards it.

4. Functions of a Repeater

✔ Extends the length of the network
✔ Boosts weak signals
✔ Removes noise
✔ Works at Physical Layer (Layer 1) of OSI model
✔ Connects two segments of the same LAN

5. Features (for short notes)

- Does not filter traffic

- Does not understand MAC addresses

- Cannot connect different types of networks

- Only regenerates signals

- Works only in wired LANs

6. Where Repeaters are Used?

- Long Ethernet cables (beyond 100 meters limit)

- Older coaxial LANs

- To extend Wi-Fi coverage (Wi-Fi repeaters)

- Campus or building LAN extensions

7. Advantages of Repeaters

✔ Simple and inexpensive
✔ Extends network distance
✔ Improves signal quality
✔ Easy to install

8. Disadvantages of Repeaters

✘ Cannot reduce network congestion
✘ Cannot connect different LAN types
✘ Repeaters repeat all signals (including noise)
✘ Useful only in small networks

9. Difference Between Repeater and Bridge (Very important)

| Repeater | Bridge |
| --- | --- |
| Works at Physical Layer | Works at Data Link Layer |
| Regenerates signals | Filters traffic using MAC address |
| Extends same LAN only | Connects different LANs |
| No traffic control | Reduces congestion |
| Cheap | Costlier |

10. Short Summary for Quick Revision

Connecting LANs:

Process of linking multiple LANs to communicate as one larger network.

Backbone Network:

A high-speed central network connecting multiple LANs across floors/buildings.

Repeater:

A Layer 1 device that amplifies and regenerates signals to extend network distance.

1. What is a Router?

A router is a networking device that connects multiple networks and directs data packets from one network to another.

It works at the Network Layer (Layer 3) of the OSI model.

Example:
Home Router connects LAN ↔ Internet.

2. Main Function of a Router

A router’s job is to:

✔ Receive data packets

✔ Read their IP addresses

✔ Choose the best path

✔ Forward them to the correct network

This process is called Routing.

3. Why Routers Are Used?

Routers are used to:

- Connect different networks

- Provide internet access

- Reduce network traffic

- Increase security

- Divide network into subnets

4. How Router Works (Step-by-Step)

- Router receives a packet.

- Checks the destination IP address.

- Searches the routing table.

- Chooses the best path (shortest/higher priority).

- Sends the packet to the next router or final destination.

5. Routing Table

A routing table contains:

- Destination network

- Next hop

- Cost/metric

- Interface

Routers update routing tables automatically using routing protocols.

6. Types of Routing

1) Static Routing

- Routes manually entered by admin

- Simple but not automatic

2) Dynamic Routing

- Uses routing protocols

- Automatically updates routes

Examples of Routing Protocols:

- RIP (Routing Information Protocol)

- OSPF (Open Shortest Path First)

- BGP (Border Gateway Protocol)

- EIGRP

7. Features of Router

✔ Works on IP addresses
✔ Connects different networks
✔ Provides security and firewall features
✔ Supports VLANs & subnetting
✔ Assigns IPs via DHCP (in home routers)
✔ Uses NAT to translate private ↔ public IP

8. Functions of Router (Important for Exams)

(a) Forwarding

Sending packets to the correct path.

(b) Routing

Finding the best path using algorithms.

(c) Packet Filtering

Checking and blocking unwanted traffic.

(d) Network Address Translation (NAT)

Allowing many devices to share one public IP.

(e) DHCP

Assigning IP addresses automatically.

9. Where Routers Are Used?

- Homes

- Colleges & offices

- Internet Service Providers

- Data centers

- WAN connections

- Large enterprises

10. Advantages of Routers

✔ Connects different LANs/WANs
✔ Controls congestion
✔ Provides high security
✔ Reduces broadcast traffic
✔ Supports dynamic routing
✔ Efficient data traffic management

11. Disadvantages of Routers

✘ Slower compared to switches
✘ Expensive
✘ Complex configuration
✘ Requires more processing power

12. Difference Between Switch and Router

| Switch | Router |
| --- | --- |
| Works at Layer 2 | Works at Layer 3 |
| Uses MAC address | Uses IP address |
| Connects devices in a LAN | Connects different networks |
| No routing table | Uses routing table |
| Faster, less intelligence | Slower, more intelligent |

13. Difference Between Hub and Router

| Hub | Router |
| --- | --- |
| Layer 1 | Layer 3 |
| Broadcasts all data | Sends data to correct network |
| No IP understanding | Understands IP addresses |
| Low security | High security |

14. Simple Diagram

LAN 1 ----\

\

Router ---- Internet

/

LAN 2 ----/

Router connects many LANs and provides internet access.

Unit 6